The best offline speech recognition model for real-time drone control is Vosk.

While OpenAI's Whisper is more accurate for transcription, Vosk is superior for drone control because:

- Latency: Vosk processes audio in real-time streaming (milliseconds), whereas Whisper typically processes chunks (seconds of delay). You need the drone to stop instantly when you say "Stop."

- Lightweight: It runs easily on standard CPUs without heavy GPU requirements.

- Installation: It installs via pip with pre-compiled binaries, avoiding the complex compilation issues often found with other engines.

### How to Install (Mac & Windows "Native" Method)

The biggest hurdle in Python speech recognition is installing the microphone library (pyaudio), which often requires complex system compilers (Homebrew, Visual Studio).

The "Native" solution is to use SoundDevice. It comes with pre-built binaries (DLLs/dylibs) for Windows and macOS, so it usually "just works" without installing system dependencies.

#### Step 1: Install Python Libraries

Open your terminal (Mac) or Command Prompt (Windows) and run:

Bash

pip install vosk sounddevice djitellopy numpy

#### Step 2: Download the Offline Model

Vosk needs a small language model folder to work.

- Download the Vosk Small English Model (approx. 40MB): vosk-model-small-en-us-0.15.zip

- Unzip the file.

- Rename the folder to vosk_model (it should contain files like am, conf, graph).

- Place the vosk_model folder inside your project folder.

#### Step 3: The Universal Control Script

This script works on both macOS and Windows. It uses sounddevice to capture raw audio and feeds it directly into vosk, bypassing the troublesome SpeechRecognition and PyAudio libraries entirely.

Save this as control.py:

Python

import sys
import json
import queue
import sounddevice as sd
from vosk import Model, KaldiRecognizer
from djitellopy import Tello

# --- Configuration ---
# Tello flight speed (cm/s)
SPEED = 30
# Model path (must be in the same folder)
MODEL_PATH = " vosk_model " 

# --- Initialize Tello ---
print("[INFO] Connecting to Tello...")
tello = Tello()

try:
    tello.connect()
    tello.streamoff()
    print(f"[SUCCESS] Battery: {tello.get_battery()}%")
except Exception as e:
    print(f"[WARN] Tello connection failed: {e}")
    print("      (Running in Simulation Mode)")
    tello = None

# --- Audio Setup ---
# A thread-safe queue to pass audio from the callback to the main loop
q = queue.Queue()

def callback(indata, frames, time, status):
    """This is called (from a separate thread) for each audio block."""
    if status:
        print(status, file=sys.stderr)
    q.put(bytes(indata))

# --- Command Execution ---
def parse_and_execute(text):
    if not text: return
    
    print(f" >> Heard: '{text}'")
    
    if not tello: return # Skip if simulating

    try:
        if "take off" in text: tello.takeoff()
        elif "land" in text: tello.land()
        elif "up" in text: tello.move_up(SPEED)
        elif "down" in text: tello.move_down(SPEED)
        elif "left" in text: tello.move_left(SPEED)
        elif "right" in text: tello.move_right(SPEED)
        elif "forward" in text: tello.move_forward(SPEED)
        elif "back" in text: tello.move_back(SPEED)
        elif "stop" in text: tello.send_rc_control(0,0,0,0) # Emergency hover
        elif "flip" in text: tello.flip_forward()
    except Exception as e:
        print(f"Command Error: {e}")

# --- Main Loop ---
try:
    print(f"[INFO] Loading Vosk Model from '{MODEL_PATH}'...")
    model = Model(MODEL_PATH)
    rec = KaldiRecognizer(model, 16000)

    print("\n" + "="*40)
    print("  VOSK SYSTEM READY (Say 'Take Off')")
    print("="*40 + "\n")

    # Start the microphone stream
    # dtype='int16', channels=1, samplerate=16000 are required for Vosk
    with sd.RawInputStream(samplerate=16000, blocksize=8000, device=None,
                           dtype='int16', channels=1, callback=callback):
        while True:
            data = q.get()
            if rec.AcceptWaveform(data):
                # Valid command phrase completed
                result = json.loads(rec.Result())
                parse_and_execute(result.get("text", ""))
            else:
                # Partial result (optional: use for even faster reaction)
                # partial = json.loads(rec.PartialResult())
                # print(partial["partial"]) 
                pass

except KeyboardInterrupt:
    print("\nDone.")
except Exception as e:
    print(f"\n[Error] {e}")
    print("Did you forget to download and rename the 'model' folder?")

To control the Tello drone using OpenWakeWord (for the wake phrase) and Vosk (for offline commands) using the "native" audio libraries on both Windows and macOS, follow these steps.

### The Architecture

- OpenWakeWord listens for "Hey Jarvis" (efficient, low latency).

- Vosk (Offline STT) listens for the command "Take Off" (high vocabulary).

- SoundDevice captures audio without needing complex system installations like pyaudio.

### Step 1: Install Python Libraries (Windows & Mac)

Open your Terminal (Mac) or Command Prompt (Windows). We will use sounddevice which usually includes the necessary audio binaries, avoiding the need for Homebrew or Visual Studio build tools.

Bash

pip install sounddevice numpy openwakeword vosk djitellopy

### Step 2: Download Models (Must be done before connecting to Tello)

Since the Tello drone uses Wi-Fi, you will lose internet access once connected. You must download the AI models now while you still have home internet.

- Create a folder for your project (e.g., TelloControl).

- Download Vosk Model (for commands):

  - Download vosk-model-small-en-us-0.15.zip.

  - Unzip it and rename the folder to vosk_model.

  - Place it inside your TelloControl folder.

- Download OpenWakeWord Models:

  - Create a simple Python script named download_models.py and run it once:

Python

import openwakeword
openwakeword.utils.download_models()
print("OpenWakeWord models downloaded!")

This saves the models (like hey_jarvis_v0.1.tflite) to your Python site-packages automatically.

### Step 3: The Control Script (Mac & Windows)

Save this code as main.py inside your TelloControl folder.

This script uses SoundDevice to capture raw audio chunks. It feeds them into OpenWakeWord. Once the wake word is detected, it switches to "Command Mode" where it feeds audio into Vosk to recognize "Take off", "Up", etc.

Python

import sys
import queue
import json
import numpy as np
import sounddevice as sd
from openwakeword.model import Model as OWWModel
from vosk import Model as VoskModel, KaldiRecognizer
from djitellopy import Tello

# --- CONFIGURATION ---
WAKE_WORD = "hey_jarvis"   # The trigger phrase
VOSK_PATH = "vosk_model"   # Path to your unzipped Vosk folder
SPEED = 30                 # Drone speed (cm/s)

# --- GLOBAL STATE ---
q = queue.Queue()
is_command_mode = False    # False = Listening for Wake Word; True = Listening for Command

# --- AUDIO CALLBACK ---
# This runs in a separate thread to capture audio continuously
def callback(indata, frames, time, status):
    if status:
        print(status, file=sys.stderr)    q.put(bytes(indata))

# --- DRONE CONTROL ---
def execute_command(text, drone):
    if not text: return
    text = text.lower()
    print(f" >> COMMAND RECOGNIZED: '{text}'")
    
    if not drone: return

    try:
        if "take off" in text: drone.takeoff()
        elif "land" in text: drone.land()
        elif "up" in text: drone.move_up(SPEED)
        elif "down" in text: drone.move_down(SPEED)
        elif "left" in text: drone.move_left(SPEED)
        elif "right" in text: drone.move_right(SPEED)
        elif "forward" in text: drone.move_forward(SPEED)
        elif "back" in text: drone.move_back(SPEED)
        elif "flip" in text: drone.flip_forward()
        elif "stop" in text: drone.send_rc_control(0,0,0,0)
    except Exception as e:
        print(f"Command Error: {e}")

# --- MAIN ---
def main():
    global is_command_mode
    
    # 1. Setup Drone
    print("[INIT] Connecting to Tello...")
    drone = Tello()
    try:
        drone.connect()
        drone.streamoff()
        print(f"[SUCCESS] Tello Battery: {drone.get_battery()}%")
    except Exception as e:
        print(f"[WARN] Tello not found. Running in Simulation Mode. Error: {e}")
        drone = None

    # 2. Load AI Models
    print(f"[INIT] Loading OpenWakeWord ({WAKE_WORD})...")
    oww = OWWModel(wakeword_models=[WAKE_WORD])
    
    print(f"[INIT] Loading Vosk ({VOSK_PATH})...")
    if not os.path.exists(VOSK_PATH):
        print(f"[ERROR] Vosk model folder '{VOSK_PATH}' not found! Download it first.")
        return
    v_model = VoskModel(VOSK_PATH)
    v_rec = KaldiRecognizer(v_model, 16000)

    # 3. Start Microphone Stream
    # OpenWakeWord expects 16-bit PCM samples at 16kHz
    print("\n" + "="*50)
    print(f"  SYSTEM READY: Say '{WAKE_WORD}' then a command.")
    print("="*50 + "\n")

    with sd.RawInputStream(samplerate=16000, blocksize=1280, dtype='int16',
                           channels=1, callback=callback):
        while True:
            # Get audio chunk
            data = q.get()
            
            if not is_command_mode:
                # --- WAKE WORD DETECTION ---
                # Convert raw bytes to numpy array for OpenWakeWord
                audio_np = np.frombuffer(data, dtype=np.int16)
                
                # Predict
                prediction = oww.predict(audio_np)
                
                if prediction[WAKE_WORD] > 0.5:
                    print(f"\n[!] WAKE WORD DETECTED! Speak now...")
                    is_command_mode = True
                    v_rec.Reset() # Clear previous buffer
            else:
                # --- COMMAND RECOGNITION (VOSK) ---
                if v_rec.AcceptWaveform(data):
                    # Full phrase recognized
                    result = json.loads(v_rec.Result())
                    cmd_text = result.get("text", "")
                    
                    if cmd_text:
                        execute_command(cmd_text, drone)
                        print("[INFO] Returning to Wake Word mode...")
                        is_command_mode = False
                        oww.reset() # Reset wake word buffer
                else:
                    # Partial result (optional print)
                    pass

import os
if __name__ == "__main__":
    try:
        main()
    except KeyboardInterrupt:
        print("\nStopping...")

### How to Run

- Disconnect your computer from your home Wi-Fi.

- Connect your computer Wi-Fi to the Tello drone (Network: TELLO-XXXXXX).

- Run the script:

Bash

python main.py

- Say "Hey Jarvis".

  - Console: [!] WAKE WORD DETECTED! Speak now...

- Say "Take Off".

  - Console: >> COMMAND RECOGNIZED: 'take off' (Drone takes off).

Using PocketSphinx for drone control is a classic choice for offline voice recognition. However, installing it can be tricky because it requires compilation.

To satisfy your request for "native audio" (avoiding the complex PyAudio installation), we will use the sounddevice library. It comes with pre-built binaries for Windows and macOS, making it much easier to set up than standard microphone libraries.

### Step 1: Install Dependencies (The "Hard" Part)

PocketSphinx requires some system tools to compile during installation.

For Windows:

- Install C++ Build Tools: You likely need "Microsoft Visual C++ 14.0 or greater". Download the Visual Studio Build Tools, run the installer, and select "Desktop development with C++".

- Install Python Libraries: Open Command Prompt and run:

Bash

pip install sounddevice djitellopy SpeechRecognition

- Install PocketSphinx:

Bash

pip install pocketsphinx

(If this fails on Windows, try downloading the Unofficial Windows Binaries .whl file and pip installing that file directly).

For macOS:

- Install SWIG (Crucial): macOS cannot install PocketSphinx without swig. You need Homebrew.

Bash

brew install swig portaudio

- Install Python Libraries:

Bash

pip install sounddevice djitellopy SpeechRecognition pocketsphinx

### Step 2: The "Native Audio" Control Script

This script uses a "Hybrid" approach:

- sounddevice: Captures audio natively (bypassing PyAudio errors).

- SpeechRecognition: Wraps the complicated PocketSphinx API.

- djitellopy: Controls the drone.

Create a file named sphinx_native.py:

Python

import time
import queue
import sounddevice as sd
import speech_recognition as sr
from djitellopy import Tello

# --- CONFIGURATION ---
SPEED = 30       # Drone speed in cm/s
DURATION = 3.5   # How long to listen for a command (seconds)
SAMPLE_RATE = 16000

# --- DRONE SETUP ---
print("[INFO] Connecting to Tello...")
tello = Tello()try:
    tello.connect()
    tello.streamoff()
    print(f"[SUCCESS] Battery: {tello.get_battery()}%")
except Exception as e:
    print(f"[WARN] Connection failed (Simulating). Error: {e}")
    tello = None

# --- COMMAND MAPPING ---
def execute_command(text):
    if not text: return
    text = text.lower()
    print(f" >> RECOGNIZED: '{text}'")
    
    if not tello: return

    try:
        # Note: PocketSphinx often struggles with short words like "up".
        # We use longer phrases to improve accuracy.
        if "take off" in text or "takeoff" in text: tello.takeoff()
        elif "land" in text: tello.land()
        elif "go up" in text: tello.move_up(SPEED)
        elif "go down" in text: tello.move_down(SPEED)
        elif "go left" in text: tello.move_left(SPEED)
        elif "go right" in text: tello.move_right(SPEED)
        elif "go forward" in text: tello.move_forward(SPEED)
        elif "go back" in text: tello.move_back(SPEED)
        elif "flip" in text: tello.flip_forward()
        else: print(" -- Command not mapped")
    except Exception as e:
        print(f"Error: {e}")

# --- MAIN LOOP ---
print("\n" + "="*40)
print("  POCKETSPHINX CONTROL (OFFLINE)")
print("  Say: 'Take Off', 'Go Up', 'Land'...")
print("="*40 + "\n")

# Initialize Recognizer
r = sr.Recognizer()

try:
    while True:
        print("Listening...", end="", flush=True)
        
        # 1. Record Audio using SoundDevice (The Native Way)
        # We record a fixed chunk of audio (e.g., 3.5 seconds) into a numpy array
        recording = sd.rec(int(DURATION * SAMPLE_RATE), samplerate=SAMPLE_RATE, channels=1, dtype='int16')
        sd.wait() # Wait for recording to finish
        print(" Processing...")
        
        # 2. Convert Raw Audio to SpeechRecognition AudioData
        # (This bridges sounddevice -> pocketsphinx)
        audio_data = sr.AudioData(recording.tobytes(), SAMPLE_RATE, 2) # 2 bytes width = int16

        # 3. Recognize using PocketSphinx
        try:
            # recognize_sphinx works offline
            command = r.recognize_sphinx(audio_data)
            execute_command(command)
        except sr.UnknownValueError:
            # Sphinx didn't understand the audio
            pass 
        except sr.RequestError as e:
            print(f"Sphinx Error: {e}")

except KeyboardInterrupt:
    print("\nStopping...")
    if tello: tello.end()

### Step 3: Improving Accuracy (The "Dictionary" Trick)

PocketSphinx is not as smart as Siri/Google. Out of the box, it tries to match every English word, which leads to many errors (e.g., hearing "Go Up" as "Grow Pup").

To fix this, you should restrict its vocabulary.

- Locate the library: Find where pocketsphinx is installed in your Python folder.

- Or, use the keyword_entries parameter: You can modify the recognition line in the script above to look strictly for keywords. Change:

Python

command = r.recognize_sphinx(audio_data)

to:

Python

# Tuple format: (keyword, sensitivity) - sensitivity 1.0 is default
keywords = [
    ("take off", 1.0),
    ("land", 1.0),
    ("go up", 1.0), 
    ("go down", 1.0),
    ("go left", 1.0),
    ("go right", 1.0)
]
command = r.recognize_sphinx(audio_data, keyword_entries=keywords)

Note: This specific feature depends on the version of the SpeechRecognition wrapper and may require pocketsphinx-python to be built correctly with keyword search support.

To use the same model.tflite you created for the M5Core2 on a Raspberry Pi Zero 2 W, you need a script that mimics the M5Core2's behavior: capturing 1 second of audio at 8kHz, converting it to the correct format (Int8), and running the inference.

Here is the step-by-step guide and the Python code.

### Step 1: Install Dependencies

The Raspberry Pi Zero 2 W needs the TensorFlow Lite runtime (lighter than full TensorFlow) and audio libraries.

Open your RPi terminal and run:

Bash

# 1. Update system and install PortAudio (required for sounddevice)
sudo apt-get update
sudo apt-get install python3-pip libportaudio2 libatlas-base-dev

# 2. Install Python libraries
# 'tflite-runtime' is best for Pi Zero. 
# If pip cannot find tflite-runtime, use the official Google index:
pip3 install tflite-runtime --extra-index-url https://google-coral.github.io/py-repo/

# 3. Install Audio and Math libraries
pip3 install sounddevice numpy

### Step 2: The Python Inference Script

Save the following code as rpi_inference.py. Place your model.tflite file in the same folder.

This script includes a Quantization Converter to translate the high-quality float audio from the USB mic into the compressed "Int8" format that your specific model expects.

Python

import time
import numpy as np
import sounddevice as sd
import tflite_runtime.interpreter as tflite

# --- CONFIGURATION ---
MODEL_PATH = "model.tflite"
SAMPLE_RATE = 8000        # Must match your training script (8kHz)
DURATION = 1.0            # Duration in seconds
THRESHOLD = 0.5           # Confidence threshold
# Define your labels exactly as they were in your training dataset
LABELS = ["down", "land", "left", "right", "takeoff", "up"]

def main():
    # 1. Load TFLite Model
    print(f"[INFO] Loading {MODEL_PATH}...")

interpreter = tflite.Interpreter(model_path=MODEL_PATH)
    interpreter.allocate_tensors()

    # Get input/output details
    input_details = interpreter.get_input_details()
    output_details = interpreter.get_output_details()
    input_index = input_details[0]['index']
    output_index = output_details[0]['index']

    # Check expected input type (Int8 vs Float32)
    input_type = input_details[0]['dtype']
    print(f"[INFO] Model expects input type: {input_type}")

    # 2. Setup Audio Stream
    # We calculate the number of samples required (e.g., 8000)
    expected_samples = int(SAMPLE_RATE * DURATION)
    
    print(f"[INFO] Listening on USB Mic ({SAMPLE_RATE}Hz)...")
    print("      Press Ctrl+C to stop.")

    try:
        while True:
            # --- Capture Audio ---
            # sounddevice.rec records into a NumPy array
            # We use blocking=True to simplify the loop
            audio_buffer = sd.rec(
                frames=expected_samples, 
                samplerate=SAMPLE_RATE, 
                channels=1, 
                dtype='float32',  # Capture as float first
                blocking=True
            )
            
            # --- Preprocess ---
            # Remove extra dimensions (samples, 1) -> (samples,)
            input_data = audio_buffer.flatten()

            # CONVERSION: Float32 (-1.0 to 1.0) -> Int8 (-128 to 127)
            # Your model was trained with Int8 quantization for M5Core2
            if input_type == np.int8:
                # Scale float to int8 range
                input_data = (input_data * 127).astype(np.int8)
            elif input_type == np.float32:
                # If you trained a non-quantized model, keep as float32
                input_data = input_data.astype(np.float32)

            # Reshape to match model input (1, 8000) or (1, 8000, 1)
            # We use the shape from input_details to be safe
            model_input = np.reshape(input_data, input_details[0]['shape'])

            # --- Inference ---
            interpreter.set_tensor(input_index, model_input)
            interpreter.invoke()
            output_data = interpreter.get_tensor(output_index)

            # --- Result ---
            # output_data is usually [[prob1, prob2, ...]]
            # If model is int8, output is also int8 (-128 to 127). 
            # We must convert it back to probability (0.0 to 1.0) approx.
            
            if output_details[0]['dtype'] == np.int8:
                # Dequantize for display: (val + 128) / 255 roughly
                # Or just rely on relative magnitude
                prediction = output_data[0]
                max_idx = np.argmax(prediction)
                confidence = (prediction[max_idx] + 128) / 255.0
            else:
                prediction = output_data[0]
                max_idx = np.argmax(prediction)
                confidence = prediction[max_idx]

            # Print if confidence is high
            if confidence > THRESHOLD:
                cmd = LABELS[max_idx]
                print(f"DETECTED: {cmd.upper()} ({confidence:.2f})")
                
                # TODO: Add your drone control code here
                # if cmd == "takeoff": tello.takeoff()

    except KeyboardInterrupt:
        print("\n[INFO] Stopping...")

if __name__ == "__main__":
    main()

### Troubleshooting USB Microphones on RPi

If the script crashes with PortAudioError: Error opening InputStream, your RPi might not be using the USB microphone as the default.

- List Devices: Run this python one-liner to find your USB mic ID:

Bash

python3 -c "import sounddevice as sd; print(sd.query_devices())"

Look for the number associated with "USB Audio Device".

- Specify Device (Optional): If your USB mic is device #2, modify the sd.rec line in the script:

Python

audio_buffer = sd.rec(..., device=2)

- Low Volume? If the confidence is always low, your mic volume might be zero. Run alsamixer, press F6 to select your USB card, and raise the "Capture" volume.

This is a complex task that involves three distinct stages: Data Collection, Model Training (Python), and Embedded Inference (C++).

Since you requested to avoid Edge Impulse, we will build a "Raw Audio" pipeline. This is the most educational approach as it avoids complex C++ Digital Signal Processing (like MFCCs) by feeding raw audio waveforms directly into a 1D Convolutional Neural Network (CNN).

### Phase 1: Dataset Preparation

You need a dataset of .wav files (1 second long, 16kHz or 8kHz) organized by class.

- Create Folders:

Plaintext

dataset/
  ├── takeoff/
  ├── land/
  ├── up/
  ├── down/
  ├── left/
  ├── right/
  └── _noise/  (Record background silence)

- Record Audio:

  - Option A: Use your PC microphone to record 1-second clips for "takeoff" and "land" (approx 50-100 samples each).

  - Option B: Use the Google Speech Commands Dataset for "up", "down", "left", "right" and manually record only "takeoff" and "land".

- Format: Ensure all files are 16kHz or 8kHz, Mono, WAV.

### Phase 2: Python Training (Local)

We will train a 1D CNN that takes raw audio samples as input. This simplifies the C++ side significantly (no FFT/MFCC code needed).

train_model.py

Python

import os
import numpy as np
import tensorflow as tf
from tensorflow.keras import layers, models
import pathlib

# --- Configuration ---
DATASET_PATH = 'dataset' # Path to your folder
SAMPLE_RATE = 8000       # We downsample to 8kHz to save RAM on ESP32
INPUT_LENGTH = 8000      # 1 second @ 8kHz
BATCH_SIZE = 32
EPOCHS = 20

# 1. Load Dataset
data_dir = pathlib.Path(DATASET_PATH)
commands = np.array(tf.io.gfile.listdir(str(data_dir)))
commands = commands[commands != 'README.md']
print('Commands:', commands)

train_ds, val_ds = tf.keras.utils.audio_dataset_from_directory(

directory=data_dir,
    batch_size=BATCH_SIZE,
    validation_split=0.2,
    seed=0,
    output_sequence_length=INPUT_LENGTH,
    subset='both')

def squeeze(audio, labels):
    audio = tf.squeeze(audio, axis=-1) # Remove the last dimension
    return audio, labels

train_ds = train_ds.map(squeeze, tf.data.AUTOTUNE)
val_ds = val_ds.map(squeeze, tf.data.AUTOTUNE)

# 2. Build 1D CNN Model
# Input shape: (8000,) -> suitable for raw waveform
model = models.Sequential([
    layers.Input(shape=(INPUT_LENGTH,)),
    layers.Reshape((INPUT_LENGTH, 1)),  # Add channel dim for Conv1D
    layers.Conv1D(8, 32, activation='relu', strides=4), # Downsample quickly
    layers.MaxPooling1D(2),
    layers.Conv1D(16, 16, activation='relu', strides=2),
    layers.MaxPooling1D(2),
    layers.Conv1D(32, 8, activation='relu'),
    layers.GlobalAveragePooling1D(),
    layers.Dense(len(commands), activation='softmax')
])

model.compile(optimizer='adam', loss='sparse_categorical_crossentropy', metrics=['accuracy'])
model.fit(train_ds, validation_data=val_ds, epochs=EPOCHS)

# 3. Convert to TFLite (Quantized)
# Crucial for ESP32: Full Integer Quantization
def representative_dataset():
    for audio, label in train_ds.take(100):
        yield [audio]

converter = tf.lite.TFLiteConverter.from_keras_model(model)
converter.optimizations = [tf.lite.Optimize.DEFAULT]
converter.representative_dataset = representative_dataset
# Ensure Input/Output are int8 compatible
converter.target_spec.supported_ops = [tf.lite.OpsSet.TFLITE_BUILTINS_INT8]
converter.inference_input_type = tf.int8
converter.inference_output_type = tf.int8
tflite_model = converter.convert()

# 4. Save as C Header
with open('model.tflite', 'wb') as f:
    f.write(tflite_model)

# Use xxd to convert to array (Linux/Mac)
os.system('xxd -i model.tflite > model_data.cc')
print("Model saved to model_data.cc")

Output: You will get a model_data.cc file. Open it; it contains an array unsigned char model_tflite[].

Since xxd is a Linux command not included in Windows by default, you have three options. The best option is to use a Python script (since you already have Python installed for training) to mimic the behavior of xxd.

### Option 1: The Python Script (Recommended)

Create a file named tflite2cc.py in the same folder as your model and run it. This script reads the binary and generates the C++ array exactly like xxd -i.

File: tflite2cc.py

Python

import sys
import os

def convert_to_c_array(input_file, output_file):
    # 1. Read the binary file
    with open(input_file, 'rb') as f:
        data = f.read()

    # 2. Generate variable names based on filename
    # e.g., "model.tflite" -> "model_tflite"
    filename = os.path.basename(input_file)
    var_name = filename.replace('.', '_').replace('-', '_')
    
    # 3. Create the C source content
    c_content = []
    
    # Add includes and the array definition
    c_content.append(f'#include <cstdint>\n\n')
    c_content.append(f'// Model data for {filename}\n')
    c_content.append(f'const unsigned char {var_name}[] = {{\n')
    
    # Format bytes as hex (e.g., 0x0A, 0xFF)
    hex_lines = []
    line_width = 12  # Bytes per line
    
    for i in range(0, len(data), line_width):
        chunk = data[i:i+line_width]
        hex_chunk = ', '.join([f'0x{b:02x}' for b in chunk])
        hex_lines.append(f'  {hex_chunk}')
    
    c_content.append(',\n'.join(hex_lines))
    c_content.append('\n};\n\n')
    
    # Add the length variable
    c_content.append(f'const unsigned int {var_name}_len = {len(data)};\n')

    # 4. Write to output file
    with open(output_file, 'w') as f:
        f.write(''.join(c_content))
    
    print(f"Success! Converted '{input_file}' to '{output_file}'")

if __name__ == "__main__":
    # Usage: python tflite2cc.py model.tflite
    if len(sys.argv) < 2:
        print("Usage: python tflite2cc.py <model.tflite>")
    else:
        input_path = sys.argv[1]
        output_path = input_path.rsplit('.', 1)[0] + "_data.cc"
        convert_to_c_array(input_path, output_path)

How to run it:

PowerShell

python tflite2cc.py model.tflite

This produces model_data.cc containing unsigned char model_tflite[].

### Option 2: Using Git Bash (If installed)

If you have Git installed on Windows, you likely have xxd already available in the "Git Bash" terminal.

- Right-click your folder and select "Open Git Bash here".

- Run the standard command:

Bash

xxd -i model.tflite > model_data.cc

### Phase 3: M5Core2 Implementation (Arduino)

#### 1. Dependencies

Install these libraries in your platformio.ini or Arduino Library Manager:

- M5Unified (for Mic/Hardware abstraction)

- M5Core2

- TensorFlowLite_ESP32 (Search for the library by Tanmay or similar, or install the official Arduino_TensorFlowLite and manually move it if needed. The EloquentTinyML library is the easiest wrapper if you want to skip manual TFLM setup).

Recommendation: Use EloquentTinyML for simplicity, or standard TensorFlowLite_ESP32.

#### 2. The Code (main.cpp)

This code sets up the M5Core2 microphone, records 1 second of audio into a buffer, downsamples it to 8kHz, converts it to int8, and runs inference.

C++

#include <M5Unified.h>
#include <TensorFlowLite_ESP32.h>
#include "tensorflow/lite/micro/all_ops_resolver.h"
#include "tensorflow/lite/micro/micro_interpreter.h"
#include "tensorflow/lite/micro/system_setup.h"
#include "tensorflow/lite/schema/schema_generated.h"

// --- Import Model Data ---
// Copy the content of model_data.cc here
extern const unsigned char model_tflite[]; 
extern const int model_tflite_len;

// --- Configuration ---
#define SAMPLE_RATE 8000
#define INPUT_LENGTH 8000 
#define RECORD_TIME 1000 // 1 second

// TFLite Globals
const tflite::Model* model = nullptr;
tflite::MicroInterpreter* interpreter = nullptr;
tflite::ErrorReporter* error_reporter = nullptr;
constexpr int kTensorArenaSize = 40 * 1024; // Adjust based on model size
uint8_t tensor_arena[kTensorArenaSize];

// Audio Buffer
int16_t raw_audio[INPUT_LENGTH]; // Buffer for recording
int8_t *model_input_buffer = nullptr;

// Commands (Must match the Python order alphabetical usually!)
const char* WORDS[] = {"down", "land", "left", "right", "takeoff", "up"};

void setup() {
    auto cfg = M5.config();
    M5.begin(cfg);
    M5.Log.setLogLevel(m5::log_target_serial, ESP_LOG_INFO);
    
    M5.Display.setTextSize(2);
    M5.Display.print("Init TFLite...");

    // 1. Initialize Microphone
    auto mic_cfg = M5.Mic.config();
    mic_cfg.sample_rate = SAMPLE_RATE; 
    mic_cfg.noise_filter_level = 0; // Set higher if noisy
    M5.Mic.config(mic_cfg);
    M5.Mic.begin();

    // 2. Initialize TFLite
    model = tflite::GetModel(model_tflite);
    static tflite::MicroMutableOpResolver<10> resolver;
    resolver.AddConv2D();
    resolver.AddReshape();
    resolver.AddFullyConnected();
    resolver.AddSoftmax();
    resolver.AddRelu();
    resolver.AddQuantize();
    resolver.AddDequantize();
    // Add other ops if your model uses them (check standard TFLM errors)

    static tflite::MicroInterpreter static_interpreter(
        model, resolver, tensor_arena, kTensorArenaSize, nullptr);
    interpreter = &static_interpreter;

    interpreter->AllocateTensors();
    model_input_buffer = interpreter->input(0)->data.int8;

    M5.Display.println("Ready!");
}

void loop() {
    M5.update();

    // Simple Threshold Trigger (Voice Activity Detection)
    // Only record if volume is loud enough to save battery/cpu
    // Or just loop continuously for this demo:
    
    if (M5.Mic.isEnabled()) {
        // Record 1 second (Blocking for simplicity)
        // M5Unified Mic record fills the buffer
        if (M5.Mic.record(raw_audio, INPUT_LENGTH, SAMPLE_RATE)) {
             // Wait for recording to complete
             while (M5.Mic.isRecording()) {
                 delay(1);
             }

             // 3. Preprocess: Convert int16 audio to int8 for the model
             // The model expects int8. 
             // int16 range: -32768 to 32767
             // int8 range: -128 to 127
             for (int i = 0; i < INPUT_LENGTH; i++) {
                 model_input_buffer[i] = (int8_t)(raw_audio[i] >> 8); 
             }

             // 4. Inference
             TfLiteStatus invoke_status = interpreter->Invoke();
             if (invoke_status != kTfLiteOk) {
                 M5.Display.println("Invoke failed!");
                 return;
             }

             // 5. Output Processing
             int8_t* output = interpreter->output(0)->data.int8;
             
             // Find max probability
             int max_idx = 0;
             int8_t max_val = -128;
             for (int i = 0; i < 6; i++) { // 6 classes
                 if (output[i] > max_val) {
                     max_val = output[i];
                     max_idx = i;
                 }
             }

             // Threshold check (approx 0.6 confidence in int8 scale)
             // -128 to 127 mapping: 0 is approx 50%. 
             if (max_val > 0) { 
                 M5.Display.fillScreen(BLACK);
                 M5.Display.setCursor(10, 50);
                 M5.Display.printf("CMD: %s", WORDS[max_idx]);
                 M5.Display.printf("\nConf: %d", max_val);
                 
                 // TODO: Insert UDP Send Code here to control Tello
             }
        }
    }
    delay(100);
}

### Critical Implementation Details

- Sampling Rate Matching: The Python script uses 8000 Hz. The M5Core2 Mic Config mic_cfg.sample_rate must be 8000. If they mismatch, the pitch will be shifted, and recognition will fail.

- Int8 Conversion: The line model_input_buffer[i] = (int8_t)(raw_audio[i] >> 8); is a crude but effective way to convert 16-bit audio (standard I2S) to 8-bit signed integer (what the quantized model expects).

  - Note: Real quantization requires (value / scale) + zero_point. However, for raw audio centered at 0, bit-shifting is usually sufficient for a demo.

- Inference Speed: A 1D CNN with INPUT_LENGTH=8000 is moderately heavy. If inference takes >500ms, you may need to reduce the input sample rate to 4kHz or reduce the model depth in Python.

- Integration with Tello: Once WORDS[max_idx] is detected, simply call the UDP send function (from your previous Tello code examples) to send "takeoff", "rc 0 0 30 0", etc.

To download the Google Speech Commands Dataset (Version 2) and filter it for only the specific words you requested (on, up, down, left, right, off, background_noise), you can use the script below.

This script uses the standard requests and tarfile libraries to download the archive and selectively extract only the folders you need, saving disk space.

### Python Script (download_gsc.py)

Python

import os
import tarfile
import requests
import io
from tqdm import tqdm

# --- Configuration ---
# Official URL for Google Speech Commands v0.02
DATASET_URL = "http://download.tensorflow.org/data/speech_commands_v0.02.tar.gz"
OUTPUT_DIR = "google_speech_commands_filtered"

# The specific words you want to keep
TARGET_WORDS = ["on", "up", "down", "left", "right", "off", "_background_noise_"]

def download_and_extract(url, target_words, output_dir):
    # Create output directory
    if not os.path.exists(output_dir):
        os.makedirs(output_dir)
        print(f"[INFO] Created directory: {output_dir}")
    
    print(f"[INFO] Downloading dataset from {url}...")
    
    # Stream the download to avoid memory issues (it's ~2.4GB)
    response = requests.get(url, stream=True)
    
    if response.status_code == 200:
        # Get total file size for progress bar
        total_size = int(response.headers.get('content-length', 0))
        
        # Load the stream into a TarFile object
        # We use BytesIO to wrap the raw stream so tarfile can read it
        # Note: This loads the archive into RAM or temp storage depending on implementation. 
        # For strictly low-RAM systems, it's safer to download to a temp file first.
        # Below is a "Download to File then Extract" approach for stability.
        
        temp_filename = "speech_commands_temp.tar.gz"
        
        with open(temp_filename, 'wb') as f, tqdm(
            desc="Downloading",
            total=total_size,
            unit='iB',
            unit_scale=True,
            unit_divisor=1024,
        ) as bar:
            for data in response.iter_content(chunk_size=1024):
                size = f.write(data)
                bar.update(size)
                
        print("\n[INFO] Download complete. Extracting specific folders...")
        
        # Open the downloaded tar file
        with tarfile.open(temp_filename, "r:gz") as tar:
            # Filter members
            members_to_extract = []
            for member in tar.getmembers():
                # The tar file structure is flat: "word/filename.wav"
                # We check if the file starts with one of our target words
                for word in target_words:
                    if member.name.startswith(word + "/"):
                        members_to_extract.append(member)
                        break
            
            # Extract only the matching members
            tar.extractall(path=output_dir, members=members_to_extract)
            print(f"[SUCCESS] Extracted {len(members_to_extract)} files to '{output_dir}'")
        
        # Cleanup temp file
        os.remove(temp_filename)
        print("[INFO] Temporary file removed.")
        
    else:
        print(f"[ERROR] Failed to download. Status code: {response.status_code}")

if __name__ == "__main__":
    # Note: Google uses "_background_noise_" with underscores in the tar file
    # We ensure our target list matches the folder names exactly.
    # Standard words: 'up', 'down' etc.
    # Noise folder: '_background_noise_'
    
    print(f"Targeting words: {TARGET_WORDS}")
    download_and_extract(DATASET_URL, TARGET_WORDS, OUTPUT_DIR)

### Instructions

- Install Requirements: You will need requests and tqdm (for the progress bar).

Bash

pip install requests tqdm

- Run the Script:

Bash

python download_gsc.py

- Result: You will see a folder named google_speech_commands_filtered containing only the subfolders for the words you requested.

To use the same model.tflite you created for the M5Core2 on a Raspberry Pi Zero 2 W, you need a script that mimics the M5Core2's behavior: capturing 1 second of audio at 8kHz, converting it to the correct format (Int8), and running the inference.

Here is the step-by-step guide and the Python code.

### Step 1: Install Dependencies

The Raspberry Pi Zero 2 W needs the TensorFlow Lite runtime (lighter than full TensorFlow) and audio libraries.

Open your RPi terminal and run:

Bash

# 1. Update system and install PortAudio (required for sounddevice)
sudo apt-get update
sudo apt-get install python3-pip libportaudio2 libatlas-base-dev

# 2. Install Python libraries
# 'tflite-runtime' is best for Pi Zero. 
# If pip cannot find tflite-runtime, use the official Google index:
pip3 install tflite-runtime --extra-index-url https://google-coral.github.io/py-repo/

# 3. Install Audio and Math libraries
pip3 install sounddevice numpy

### Step 2: The Python Inference Script

Save the following code as rpi_inference.py. Place your model.tflite file in the same folder.

This script includes a Quantization Converter to translate the high-quality float audio from the USB mic into the compressed "Int8" format that your specific model expects.

Python

import time
import numpy as np
import sounddevice as sd
import tflite_runtime.interpreter as tflite

# --- CONFIGURATION ---
MODEL_PATH = "model.tflite"
SAMPLE_RATE = 8000        # Must match your training script (8kHz)
DURATION = 1.0            # Duration in seconds
THRESHOLD = 0.5           # Confidence threshold
# Define your labels exactly as they were in your training dataset
LABELS = ["down", "land", "left", "right", "takeoff", "up"]

def main():
    # 1. Load TFLite Model
    print(f"[INFO] Loading {MODEL_PATH}...")

interpreter = tflite.Interpreter(model_path=MODEL_PATH)
    interpreter.allocate_tensors()

    # Get input/output details
    input_details = interpreter.get_input_details()
    output_details = interpreter.get_output_details()
    input_index = input_details[0]['index']
    output_index = output_details[0]['index']

    # Check expected input type (Int8 vs Float32)
    input_type = input_details[0]['dtype']
    print(f"[INFO] Model expects input type: {input_type}")

    # 2. Setup Audio Stream
    # We calculate the number of samples required (e.g., 8000)
    expected_samples = int(SAMPLE_RATE * DURATION)
    
    print(f"[INFO] Listening on USB Mic ({SAMPLE_RATE}Hz)...")
    print("      Press Ctrl+C to stop.")

    try:
        while True:
            # --- Capture Audio ---
            # sounddevice.rec records into a NumPy array
            # We use blocking=True to simplify the loop
            audio_buffer = sd.rec(
                frames=expected_samples, 
                samplerate=SAMPLE_RATE, 
                channels=1, 
                dtype='float32',  # Capture as float first
                blocking=True
            )
            
            # --- Preprocess ---
            # Remove extra dimensions (samples, 1) -> (samples,)
            input_data = audio_buffer.flatten()

            # CONVERSION: Float32 (-1.0 to 1.0) -> Int8 (-128 to 127)
            # Your model was trained with Int8 quantization for M5Core2
            if input_type == np.int8:
                # Scale float to int8 range
                input_data = (input_data * 127).astype(np.int8)
            elif input_type == np.float32:
                # If you trained a non-quantized model, keep as float32
                input_data = input_data.astype(np.float32)

            # Reshape to match model input (1, 8000) or (1, 8000, 1)
            # We use the shape from input_details to be safe
            model_input = np.reshape(input_data, input_details[0]['shape'])

            # --- Inference ---
            interpreter.set_tensor(input_index, model_input)
            interpreter.invoke()
            output_data = interpreter.get_tensor(output_index)

            # --- Result ---
            # output_data is usually [[prob1, prob2, ...]]
            # If model is int8, output is also int8 (-128 to 127). 
            # We must convert it back to probability (0.0 to 1.0) approx.
            
            if output_details[0]['dtype'] == np.int8:
                # Dequantize for display: (val + 128) / 255 roughly
                # Or just rely on relative magnitude
                prediction = output_data[0]
                max_idx = np.argmax(prediction)
                confidence = (prediction[max_idx] + 128) / 255.0
            else:
                prediction = output_data[0]
                max_idx = np.argmax(prediction)
                confidence = prediction[max_idx]

            # Print if confidence is high
            if confidence > THRESHOLD:
                cmd = LABELS[max_idx]
                print(f"DETECTED: {cmd.upper()} ({confidence:.2f})")
                
                # TODO: Add your drone control code here
                # if cmd == "takeoff": tello.takeoff()

    except KeyboardInterrupt:
        print("\n[INFO] Stopping...")

if __name__ == "__main__":
    main()

### Troubleshooting USB Microphones on RPi

If the script crashes with PortAudioError: Error opening InputStream, your RPi might not be using the USB microphone as the default.

- List Devices: Run this python one-liner to find your USB mic ID:

Bash

python3 -c "import sounddevice as sd; print(sd.query_devices())"

Look for the number associated with "USB Audio Device".

- Specify Device (Optional): If your USB mic is device #2, modify the sd.rec line in the script:

Python

audio_buffer = sd.rec(..., device=2)

- Low Volume? If the confidence is always low, your mic volume might be zero. Run alsamixer, press F6 to select your USB card, and raise the "Capture" volume.