Raspberry Pi Bookworm OS (32-bit) which uses the new libcamera stack (accessed via rpicam-apps or the Picamera2 Python library). The legacy raspivid/raspistill commands will not work.

### Prerequisites

- Hardware: RPi Zero W2 with CSI Camera connected.

- OS: Raspberry Pi OS Bookworm (32-bit).

- Network: RPi and Host (Mac/PC) must be on the same WiFi.

- Dependencies (RPi):

Bash

sudo apt update
sudo apt install rpicam-apps python3-libcamera python3-kms++ libcamera-ipa
# Install Picamera2 for Python access
sudo apt install python3-picamera2

- Dependencies (Host PC - Mac/Windows):

  - VLC Media Player: Installed.

  - Python Libraries:

Bash

pip install opencv-python ultralytics numpy

### Part 1: Single Image Transfer

#### Example A: Using rpicam-jpeg (Command Line)

We will capture an image on the RPi and instantly send it to the Host using netcat (nc).

- On Host PC (Receiver): Open a terminal (Command Prompt/PowerShell on Windows, Terminal on Mac) and listen for data.

  - Mac: nc -l 5000 > received_image.jpg

  - Windows: Netcat is not native. Use PowerShell to "pull" the file after capture, or use the Python method below. (For strict CLI on Windows, you'd enable SCP).

Install Ncat with the Nmap suite (Recommended)

Ncat is integrated with the Nmap suite and is the modern, more secure and feature-rich successor to the original Netcat.

- Download the Nmap installer: Go to the official Nmap download page and download the latest stable Windows self-installer (e.g., nmap-*-setup.exe).

- Run the installer: Execute the downloaded .exe file.

- Select components: During the installation process, ensure that the Ncat and Register Nmap Path options are selected. This adds Ncat to your system's PATH, allowing you to run it from any command prompt location.

- Complete installation: Follow the prompts to finish the installation.

- Verify installation: Open a new Command Prompt or PowerShell window and type ncat --version. This should display the installed Ncat version if the installation was successful.

You will use the command ncat instead of nc when using this version.

- On RPi (Sender): Run this command to capture and pipe the output directly to the host IP.

Bash

# Replace HOST_IP with your laptop's IP address (e.g., 192.168.1.10)
rpicam-jpeg -o - -t 2000 --width 1920 --height 1080 | nc HOST_IP 5000

  - -o -: Output to standard output (stdout).

  - -t 2000: 2-second preview delay for auto-exposure.

#### Example B: Using Python (Picamera2)

This script captures a standard JPEG and sends it over a TCP socket.

RPi Sender (send_image.py):

Python

import socket
import time
from picamera2 import Picamera2

# CONFIG
HOST_IP = '192.168.1.10' # REPLACE WITH YOUR HOST PC IP
PORT = 65432

# Setup Camera
picam2 = Picamera2()
config = picam2.create_preview_configuration(main={"size": (1920, 1080), "format": "XRGB8888"})
picam2.configure(config)
picam2.start()

# Warmup
time.sleep(2)

# Capture JPEG data in memory
jpg_data = picam2.capture_buffer("main", format="jpeg")

# Send via Socket
with socket.socket(socket.AF_INET, socket.SOCK_STREAM) as s:
    s.connect((HOST_IP, PORT))
    s.sendall(jpg_data)
    print("Image sent!")

picam2.stop()

Host Receiver (recv_image.py):

Python

import socket

PORT = 65432

with socket.socket(socket.AF_INET, socket.SOCK_STREAM) as s:
    s.bind(('0.0.0.0', PORT))
    s.listen()
    print("Waiting for image...")
    conn, addr = s.accept()
    with conn:
        with open('received_py.jpg', 'wb') as f:
            while True:
                data = conn.recv(4096)
                if not data: break
                f.write(data)
    print("Image saved as received_py.jpg")

### Part 2: Continuous Streaming (Video)

#### Example A: Stream to VLC (rpicam-vid CLI)

This is the lowest latency method for pure viewing.

- On RPi (Sender): We start a TCP server stream on the Pi.

Bash

rpicam-vid -t 0 --inline --listen -o tcp://0.0.0.0:8554 --width 1280 --height 720 --framerate 30

  - -t 0: Record indefinitely.

  - --inline: Insert headers for streaming.

  - --listen: Act as a server.

- On Host (VLC):

  - Open VLC.

  - Go to File -> Open Network.

  - Enter URL: tcp/h264://<RPI_IP_ADDRESS>:8554

  - Click Open.

#### Example B: Stream to Python (OpenCV & YOLO)

Direct H.264 streaming to OpenCV is difficult because OpenCV often struggles to decode raw streams over TCP. The most robust method for Python-to-Python streaming is sending JPEG frames (MJPEG style) over TCP.

1. RPi Sender (Streaming JPEGs via TCP): Save as rpi_stream_sender.py.

Python

import socket
import struct
import time
import io
from picamera2 import Picamera2

# CONFIG
HOST_IP = '192.168.1.10' # REPLACE WITH HOST PC IP
PORT = 8000

# Setup Camera (Picamera2)
picam2 = Picamera2()
config = picam2.create_preview_configuration(main={"size": (640, 480), "format": "XRGB8888"})
picam2.configure(config)
picam2.start()

# Connect to Host
client_socket = socket.socket(socket.AF_INET, socket.SOCK_STREAM)
client_socket.connect((HOST_IP, PORT))
connection = client_socket.makefile('wb')

try:
    print("Streaming...")
    while True:
        # Capture standard JPEG
        # Note: capture_file is simpler for streaming loop than capture_buffer for direct bytes IO
        stream = io.BytesIO()
        picam2.capture_file(stream, format="jpeg")
        data = stream.getvalue()
        
        # Send Message: [Length of Image (4 bytes)][Image Data]
        # 'L' is unsigned long (4 bytes)
        connection.write(struct.pack('<L', len(data)))
        connection.write(data)
        
        # Simple throttle to prevent flooding (approx 30fps)
        # time.sleep(0.01) 

except KeyboardInterrupt:
    pass
finally:
    connection.close()
    client_socket.close()
    picam2.stop()

2. Host Receiver (OpenCV Only): Save as host_opencv_view.py.

Python

import socket
import struct
import cv2
import numpy as np
import io

PORT = 8000

server_socket = socket.socket(socket.AF_INET, socket.SOCK_STREAM)
server_socket.bind(('0.0.0.0', PORT))
server_socket.listen(0)

print(f"Listening on port {PORT}...")
conn, addr = server_socket.accept()
connection = conn.makefile('rb')

try:
    while True:
        # 1. Read the length of the image (4 bytes)
        image_len_data = connection.read(4)
        if not image_len_data: break
        
        image_len = struct.unpack('<L', image_len_data)[0]
        
        # 2. Read the image data based on the length
        image_stream = io.BytesIO()
        image_stream.write(connection.read(image_len))
        
        # 3. Rewind stream and decode
        image_stream.seek(0)
        data = np.frombuffer(image_stream.getvalue(), dtype=np.uint8)
        frame = cv2.imdecode(data, cv2.IMREAD_COLOR)
        
        if frame is not None:
            # --- OPEN CV PROCESSING EXAMPLE ---
            # Convert to Grayscale for demo
            gray_frame = cv2.cvtColor(frame, cv2.COLOR_BGR2GRAY)
            
            cv2.imshow('RPi Stream (Gray)', gray_frame)
            
        if cv2.waitKey(1) & 0xFF == ord('q'):
            break
            
except Exception as e:
    print(e)
finally:
    conn.close()
    server_socket.close()
    cv2.destroyAllWindows()

3. Host Receiver (YOLO Object Detection): Save as host_yolo_detect.py.

Python

import socket
import struct
import cv2
import numpy as np
import io
from ultralytics import YOLO

# Load YOLOv8 Nano model (small and fast for CPU)
model = YOLO("yolov8n.pt") 

PORT = 8000

server_socket = socket.socket(socket.AF_INET, socket.SOCK_STREAM)
server_socket.bind(('0.0.0.0', PORT))
server_socket.listen(0)

print(f"Listening for YOLO Stream on {PORT}...")
conn, addr = server_socket.accept()
connection = conn.makefile('rb')

try:
    while True:
        # Read payload length
        image_len_data = connection.read(4)
        if not image_len_data: break
        image_len = struct.unpack('<L', image_len_data)[0]
        
        # Read payload
        image_stream = io.BytesIO()
        image_stream.write(connection.read(image_len))
        
        # Decode image
        image_stream.seek(0)
        data = np.frombuffer(image_stream.getvalue(), dtype=np.uint8)
        frame = cv2.imdecode(data, cv2.IMREAD_COLOR)
        
        if frame is not None:
            # --- YOLO DETECTION ---
            # Run inference
            results = model(frame, verbose=False)
            
            # Plot results on the frame
            annotated_frame = results[0].plot()
            
            cv2.imshow('YOLOv8 Detection', annotated_frame)
            
        if cv2.waitKey(1) & 0xFF == ord('q'):
            break
            
except Exception as e:
    print(f"Error: {e}")
finally:
    conn.close()
    server_socket.close()
    cv2.destroyAllWindows()

### How to Run (Order is Important)

- For Python Streams: ALWAYS run the Host (Receiver) script first so it is listening.

- Then: Run the RPi (Sender) script.